# Real-Time Emotion Detection with CNN Hardware Acceleration

This project implements a real-time emotion detection system that uses both a deep learning-based software model and a Verilator-simulated hardware accelerator for CNN inference. It provides a hands-on demonstration of hybrid AI acceleration using Python, OpenCV, Keras, and SystemVerilog.

# Project Overview and Goals

The core objective of this project was to detect facial emotions in real-time using webcam input and to evaluate the efficiency of hardware-accelerated inference using a Verilator-simulated CNN compared to a software-based Keras model. The project successfully demonstrated real-time inference, performance comparison, and integration of open-source tools for synthesis and benchmarking.

# Software and Hardware Design Pipeline

The end-to-end pipeline involved training a CNN model on the FER-2013 dataset, capturing frames using OpenCV, preprocessing and feeding normalized grayscale images into a Verilated SystemVerilog CNN hardware accelerator, and logging inference results and performance metrics.

# Project Milestones

Week 2: Selected CNN-based emotion detection as the project algorithm. Profiling identified the 2D convolution as the primary bottleneck.
Week 3: Measured software inference times, ranging from 0.13s to 0.91s per frame.
Week 4: Designed a pipelined SystemVerilog CNN accelerator module.
Week 5: Integrated the Verilated accelerator using a C++ wrapper and Python subprocess interface.
Week 6: Benchmarked the system, recording a ~2.3× speedup: Avg. software time = 0.197s; Avg. hardware time = 0.086s.
Week 7: Synthesized the accelerator using OpenLane. Achieved 500 MHz max frequency, zero slack, and retrieved logic utilization and area metrics.
Week 8–10: Final integration, verification, reporting, and presentation.

# File & Directory Structure

• emotion_detector_with_model.py: Captures webcam input, processes image, runs software or hardware inference, and logs results.
• emotion_model.h5: Trained Keras CNN model.
• emotionmodel_cnn.py: Model architecture.
• train_emotion_cnn.py: Trains the CNN using the FER-2013 dataset.
• cnn_accelerator.sv: SystemVerilog CNN accelerator.
• sim_main.cpp: C++ testbench for Verilated simulation.
• input_image.txt: Image input for Verilator.
• emotion_predictions.csv / inference_log.csv: Logs inference outputs and performance.
• analyze_emotion_results.py: Visualization of speedup and prediction accuracy.
• verilator_test.py: Verilator testbench utility.
• synthesis_reports/: Area, power, and timing metrics from OpenLane.

# Key Achievements and Metrics

• Verified CNN hardware accelerator inference results against software baseline.
• Logged 100 webcam frames and validated accuracy and performance across both pipelines.
• Hardware averaged 0.086s/frame; software averaged 0.197s/frame.
• Achieved a 2.3× speedup through hardware acceleration.
• Synthesized the hardware using OpenLane. Achieved:
    - Max Clock Frequency: 500 MHz
    - Slack: 0.00 ns
    - Estimated Cell Area and Gate Count reported

# Weekly Challenges and Impact

The course's 10 weekly challenges and code fests were instrumental in developing the technical skills and critical thinking necessary for this project. Each challenge helped build a strong foundation in AI/ML hardware concepts and accelerated design flow. These challenges significantly shaped the direction and execution of this project, providing practical exposure that translated directly into the final deliverables.

# Conclusion

This project effectively demonstrates a complete hardware/software co-design approach for accelerating AI workloads. By leveraging Verilator for simulation and OpenLane for synthesis, it offers a reproducible and scalable method for validating and benchmarking RTL accelerators in real-time systems. It showcases real-world acceleration potential, system integration, and an impactful learning journey in the field of AI hardware.